Adaptive Robot Arm

Project Framework, Technical Design & Hardware Buying Guide

Voice control · YOLO + LiDAR vision · Stepper motor lift · Free AI/LLM layer

Version 2.0  |  March 2026  |  Prices in USD and CAD

# 1. Project Overview

This document defines the full expansion plan for an Arduino UNO + Python robot arm, targeting seniors, disabled individuals, and everyday assistive use. The existing system uses MediaPipe hand-gesture recognition over serial to control three servos. The goal is to add voice control, spatial awareness, a more powerful lift, and an AI layer that learns user habits — all at the lowest possible hardware cost.

## 1.1 Current System

- Python (Webcam-tracking.py): OpenCV + MediaPipe gesture detection, serial output.

- Arduino UNO: Servo library drives base, rotate, and claw servos. Lift servo declared but disabled.

## 1.2 Expansion Goals

- Natural voice control — 'open the claw', 'pick up the cup'.

- YOLO object detection + LiDAR for autonomous pick-and-place tasks.

- NEMA 17 stepper motor replacing the lift servo for torque and precision.

- Free or near-free AI/LLM layer for adaptive, preference-learning control.

# 2. System Architecture

| Layer | Description |
| --- | --- |
| Input | Voice mic + STT, gesture webcam (MediaPipe), vision camera (YOLO + LiDAR). All inputs operate simultaneously. |
| Orchestrator | Central Python asyncio event loop. Receives events from all inputs, routes to correct processing module. |
| Processing | Speech NLU (intent parsing), LLM/AI Model (planning, preferences), Object Detection (YOLO + depth fusion). Each emits a normalized ActionRequest. |
| Motion Planner | Converts ActionRequests into joint trajectories via IK. Applies safety limits, produces serial packet sequence. |
| Hardware | Arduino UNO drives servo array (base, rotate, claw) and stepper motor (lift). Sensor feedback loops back to orchestrator for learning. |

# 3. Free AI Options — $0 Alternatives

Yes — for a robot arm project with low request volume (a few calls per session), completely free options exist that are comparable in practical quality to a paid API. Here are the three best approaches, from simplest to most private:

## Option A — Groq Free Tier (Recommended)

Groq runs Llama 3.3 70B on custom silicon at over 300 tokens/second, faster than any paid API. The free Developer tier gives 1,000 requests/day — far more than any robot arm session will use. Quality is comparable to GPT-4 for structured command parsing tasks like this one.

- Model: llama-3.3-70b-versatile

- Free limits: 1,000 requests/day, 6,000 tokens/minute. No credit card needed.

- API is OpenAI-compatible — drop-in replacement for the paid Claude code in llm_agent.py.

Sign up: console.groq.com  |  Rate limits: console.groq.com/docs/rate-limits

## Option B — Google Gemini 2.5 Flash (Google AI Studio)

Google AI Studio gives completely free access to Gemini 2.5 Flash — Google's latest mid-tier model — with a generous 1 million token/minute limit. This is Google's recommended free prototyping environment and requires no payment method.

- Model: gemini-2.5-flash (via AI Studio)

- Free limits: 1,500 requests/day, 1M tokens/min in AI Studio. No credit card needed.

- Supports multimodal input — can accept camera frames directly for vision tasks.

Sign up: aistudio.google.com  |  Pricing: ai.google.dev/pricing

## Option C — Ollama (Fully Local, Zero Cost, Zero Data Sharing)

Ollama runs open-source models entirely on your own machine — no internet connection, no API keys, no rate limits, and no data ever leaves your computer. Ideal if privacy is a priority. Llama 3.3 8B runs on a modern CPU laptop at around 5–15 tokens/second (sufficient for command parsing). Llama 3.3 70B requires a GPU or Apple Silicon for usable speed.

- Recommended local model: llama3.2:3b (fast CPU) or qwen2.5:7b (better quality, still CPU-viable)

- API is OpenAI-compatible — same code as Groq, just change base_url to http://localhost:11434/v1.

- No account. No signup. No usage limits. Completely free forever.

Download: ollama.com  |  Model library: ollama.com/library

## Comparison Table

| Option | Cost | Quality | Speed | Best for |
| --- | --- | --- | --- | --- |
| Groq free tier | $0 | ★★★★★ | Very fast | Most users — easiest setup, best quality/speed |
| Google AI Studio | $0 | ★★★★★ | Fast | Multimodal tasks; processing camera images directly |
| Ollama local | $0 | ★★★★☆ | Moderate | Maximum privacy; no internet required |
| Anthropic API (paid) | ~$0–10/mo | ★★★★★ | Fast | Production use, complex multi-step planning |

For a robot arm that runs a handful of AI calls per session, the Groq free tier will realistically never hit its daily limit. Start there, and if you later need offline privacy, add Ollama as a fallback.

# 4. Hardware Bill of Materials — Specific Products & Links

All prices are approximate USD and CAD retail as of Q1 2026. Click the product name to open the purchase link. Canadian pricing uses a 1.38× multiplier. Items already in the build are marked 'Existing'.

| Product (click to buy) | USD | CAD ~ | Notes |
| --- | --- | --- | --- |
| ─── Already in build ─── |  |  |  |
| Arduino UNO R3 | Existing | — | Already installed |
| USB Webcam (1080p) | Existing | — | Reused for YOLO detection too |
| SG90 Servo × 3 (base, rotate, claw) | Existing | — | Pins 3, 6, 9 on Arduino |

| Phase 1 — Voice Control | USD | CAD ~ | Notes |
| --- | --- | --- | --- |
| FIFINE K669B USB Condenser Mic | $25 | ~$35 | Cardioid, plug-and-play, volume knob, tripod included. Best value entry mic. |

| Phase 2 — Stepper Motor Lift | USD | CAD ~ | Notes |
| --- | --- | --- | --- |
| STEPPERONLINE 17HS19-2004S1 NEMA 17 Stepper (2A, 59Ncm) | $13 | ~$18 | Most popular NEMA 17 for robotics. High torque, 1.8° step angle. |
| HiLetgo A4988 Stepper Driver Module with Heatsink (5-pack) | $8 | ~$11 | 5-pack — keep spares. Pre-soldered heatsink. Use only 1. |
| MEAN WELL 12V 2A DC Power Supply (RS-25-12) | $12 | ~$17 | Powers stepper independently from Arduino. Reliable brand. |
| 100µF 25V Capacitor + KW12 Limit Switches (2-pack) | $6 | ~$8 | Capacitor protects A4988 from voltage spikes. Switches are end-stops. |

| Phase 3 — YOLO + LiDAR Vision | USD | CAD ~ | Notes |
| --- | --- | --- | --- |
| Full LiDAR option: Slamtec RPLIDAR A1M8 360° 2D LiDAR (12m range) | $99 | ~$137 | Best option. 360° scan, 8000Hz, USB, Python library (rplidar-robotics). 12m range. |
| Budget option: MakerFocus TF-Luna Single-Point LiDAR (0.2–8m, UART/I2C) | $25 | ~$35 | Single-point depth only. Good enough for desktop pick-and-place. Simpler wiring. |

| Phase 4 — AI / LLM Layer | USD | CAD ~ | Notes |
| --- | --- | --- | --- |
| Groq free API — Llama 3.3 70B (sign up) | $0 | $0 | 1,000 req/day free. No credit card. Best quality. Recommended starting point. |
| Google AI Studio — Gemini 2.5 Flash (sign up) | $0 | $0 | 1,500 req/day free. Multimodal — can receive camera frames directly. |
| Ollama — fully local models (download) | $0 | $0 | No internet needed. No limits. Privacy-first. Runs llama3.2:3b on CPU. |

| Scenario | USD Total | CAD Total ~ |
| --- | --- | --- |
| Phases 1–3 + Full LiDAR (RPLIDAR A1M8) + Free AI | ~$163 | ~$225 |
| Phases 1–3 + Budget LiDAR (TF-Luna) + Free AI | ~$89 | ~$123 |
| Phase 1 + 2 only (no vision) + Free AI | ~$64 | ~$88 |

# 5. Implementation Phases

## Phase 1 — Voice Control (3–5 days)

### Libraries

- Vosk: offline STT. Model: vosk-model-small-en-us-0.15 (~40MB). Install: pip install vosk

- sounddevice: microphone capture. Install: pip install sounddevice

- pvporcupine: wake-word (say 'Hey Robot' to activate). Free tier at picovoice.ai

### Integration

The voice module runs as an asyncio coroutine, emitting the same ActionRequest the gesture module uses — no changes needed to the serial bridge or Arduino.

## Phase 2 — Stepper Motor Lift (2–3 days)

### Arduino library

- AccelStepper: Install via Arduino IDE Library Manager. Replaces Servo for the lift axis.

### Wiring

- A4988 STEP pin → Arduino D5, DIR pin → Arduino D4. VMOT → 12V supply. VDD → Arduino 5V.

- 100µF capacitor across VMOT and GND of the A4988 (mandatory — prevents voltage spikes).

- Limit switches on A0 (top) and A1 (bottom) with INPUT_PULLUP.

## Phase 3 — YOLO + LiDAR (7–10 days)

### Libraries

- ultralytics: YOLOv8. Install: pip install ultralytics

- rplidar-robotics: RPLIDAR A1 Python driver. Install: pip install rplidar-robotics

- ikpy: inverse kinematics. Install: pip install ikpy

### Calibration requirement

A one-time calibration places a known object at known positions and records the camera detection vs. actual coordinates to build the camera-to-arm transform matrix.

## Phase 4 — Free AI Layer (5–8 days)

### Groq implementation (3 lines to swap in)

from openai import OpenAI client = OpenAI(     api_key='gsk_YOUR_GROQ_KEY',     base_url='https://api.groq.com/openai/v1' ) response = client.chat.completions.create(     model='llama-3.3-70b-versatile',     messages=[{'role':'user','content': command}] )

### Ollama implementation (local, no internet)

# After: ollama pull llama3.2:3b client = OpenAI(     api_key='ollama',     base_url='http://localhost:11434/v1' ) # Same call as above — fully local, zero cost

# 6. Project File Structure

| File / Directory | Purpose |
| --- | --- |
| main.py | Entry point. Initialises all modules, starts asyncio event loop. |
| orchestrator.py | Central dispatcher. Routes events from inputs to planners. |
| inputs/gesture.py | Existing MediaPipe loop refactored into async coroutine. |
| inputs/voice.py | Mic capture, Porcupine wake-word, Vosk STT, intent parser. |
| inputs/vision.py | YOLOv8 detection + LiDAR depth fusion → emits TargetPose. |
| ai/llm_agent.py | LLM interface (Groq / Ollama / Gemini). Manages context, preferences. |
| ai/memory.db | SQLite log: (timestamp, command, outcome, joint_state). |
| motion/planner.py | IK solver (ikpy), trajectory smoother, safety limit checks. |
| motion/serial_bridge.py | Rate-limited serial writer; builds/validates packets. |
| arduino/sketch.ino | Updated: AccelStepper for lift + existing Servo for others. |
| config.py | All constants: ports, baud, mappings, IK params, LLM model. |

# 7. Recommended Build Order

| # | Phase | Est. Time | Success Criteria |
| --- | --- | --- | --- |
| 1 | Voice control | 3–5 days | 'Open claw' and 'move left' reliably trigger correct serial output; wake-word activates without false triggers. |
| 2 | Stepper motor lift | 2–3 days | Lift raises/lowers on command; limit switches stop over-travel; AccelStepper runs smoothly. |
| 3 | YOLO + LiDAR | 7–10 days | 'Pick up the cup' positions arm within ±2cm. Calibration matrix reproducible. |
| 4 | Free AI layer | 5–8 days | Multi-step command decomposed correctly; user preference persisted across sessions; ambiguous commands trigger clarifying question. |

Phases 3 and 4 can be developed in parallel after Phase 2 completes — they have no dependency on each other.

# 8. Python Dependencies

| Package | Phase | Purpose |
| --- | --- | --- |
| opencv-python | Existing | Webcam capture, frame processing |
| mediapipe | Existing | Hand landmark detection |
| pyserial | Existing | Arduino serial communication |
| vosk | 1 | Offline speech-to-text (small model ~40MB) |
| sounddevice | 1 | Microphone audio capture |
| pvporcupine | 1 | Wake-word detection (free tier at picovoice.ai) |
| ultralytics | 3 | YOLOv8 object detection |
| rplidar-robotics | 3 | RPLIDAR A1 Python driver |
| ikpy | 3 | Inverse kinematics solver for arm joint planning |
| groq OR openai | 4 | LLM client — works for Groq, Ollama, and Gemini via OpenAI-compat API |
| numpy | All | Matrix math for coordinate transforms |

# 9. Accessibility Design Principles

- Voice commands must tolerate low volume, slurred speech, and accents. Vosk models can be fine-tuned on user samples.

- Command confirmation before executing potentially dangerous actions: 'I will pick up the cup. Say yes to confirm.'

- Fallback hierarchy: voice → gesture → simple on-screen click interface.

- All trajectory speeds capped at 60% of hardware maximum by default. Users opt into higher speeds.

- Emergency stop: 'STOP' at any volume, or a two-handed gesture, immediately halts all motion.

- Session memory: the LLM stores frequently-used commands as shortcuts for users with deteriorating speech.

# 10. Summary

This document covers the complete expansion of the existing gesture-controlled robot arm to a fully adaptive, voice-guided, spatially-aware assistive system. The AI layer costs $0 using Groq's free tier (Llama 3.3 70B) or Ollama locally — no subscription required. Total new hardware runs between approximately $64 and $163 USD ($88–$225 CAD) depending on which phases you build and which LiDAR you choose.

Recommended starting point: buy the FIFINE K669B mic ($25), NEMA 17 stepper + A4988 ($21), and sign up for a free Groq account. That gets you Phases 1, 2, and 4 for under $50 USD — voice control, precise lift, and AI assistance — all before spending anything on LiDAR.

Document v2.0 — March 2026 | Links verified at time of writing; prices subject to change.